8MP WDR LightHunter VF Eyeball Network IR Camera

IPC3638SR3-DPZ

Key Features

Optics

- LightHunter

- Day/night functionality

- Optical glass window with higher light transmittance

- IR anti-reflection window to increase the infrared transmittance

- Smart IR, up to 30m (98ft) IR distance

- Up to 120dB Optical WDR (Wide Dynamic Range)

- 2D/3D DNR (Digital Noise Reduction)

Compression

- Ultra 265, H.265, H.264, MJPEG

- Triple streams

- ROI (Region of Interest)

- 9:16 corridor mode

Network

- ONVIF Conformance

Structure

- Support PoE power supply

- Wide temperature range: -40°C ~ 60°C (-40°F ~ 140°F)

- Wide voltage range of ±25%

- IP67

- 3-Axis

Specifications

|  | IPC3638SR3-DPZ |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Camera |  |  |  |  |  |  |  |  |
| Sensor | 1/2", 8.0 megapixel, progressive scan, CMOS |  |  |  |  |  |  |  |
| Lens | 2.8 ~ 12mm, AF automatic focusing and motorized zoom lens |  |  |  |  |  |  |  |
| DORI Distance | Lens (mm) |  | Detect (m) |  | Observe (m) | Recognize (m) |  | Identify (m) |
|  | 2.8 |  | 63 |  | 25.2 | 12.6 |  | 6.3 |
|  | 12 |  | 270 |  | 108 | 54 |  | 27 |
| Angle of view (H) | 114.8° ~ 47.3° |  |  |  |  |  |  |  |
| Angle of View (V) | 75.3° ~ 20.4° |  |  |  |  |  |  |  |
| Angle of View (O) | 141.2° ~ 55.8° |  |  |  |  |  |  |  |
| Adjustment angle | Pan: 0° ~ 360° |  |  | Tilt: 0° ~ 90° |  |  | Rotate: 0° ~ 360° |  |
| Shutter | Auto/Manual, 1 ~ 1/100000s |  |  |  |  |  |  |  |
| Minimum Illumination | Colour: 0.001Lux (F1.5, AGC ON) 0Lux with IR |  |  |  |  |  |  |  |
| Day/Night | IR-cut filter with auto switch (ICR) |  |  |  |  |  |  |  |
| S/N | >52dB |  |  |  |  |  |  |  |
| IR Range | Up to 30m (98ft) IR range |  |  |  |  |  |  |  |
| wavelength | 850nm |  |  |  |  |  |  |  |
| IR On/Off Control | Auto/Manual |  |  |  |  |  |  |  |
| Defog | Digital Defog |  |  |  |  |  |  |  |
| WDR | 120dB |  |  |  |  |  |  |  |
| Video |  |  |  |  |  |  |  |  |
| Video Compression | Ultra 265, H.265, H.264, MJPEG |  |  |  |  |  |  |  |
| Frame Rate | Main Stream: 8MP (3840*2160), Max 20fps; 5MP (2592*1944), Max 30fps; 5MP (2944*1656), Max 30fps; 4MP (2560*1440), Max 30fps; 2MP (1920*1080), Max 60fps; Sub Stream: 2MP (1920*1080), Max 30fps; Third Stream: D1 (720*576), Max 30fps |  |  |  |  |  |  |  |
| Video Bit Rate | 128 Kbps~16 Mbps |  |  |  |  |  |  |  |
| 9:16 Corridor Mode | Supported |  |  |  |  |  |  |  |
| OSD | Up to 8 OSDs |  |  |  |  |  |  |  |
| Privacy Mask | Up to 8 areas |  |  |  |  |  |  |  |
| ROI | Up to 8 areas |  |  |  |  |  |  |  |
| Motion Detection | Up to 4 areas |  |  |  |  |  |  |  |
| Image |  |  |  |  |  |  |  |  |
| White Balance |  | Auto/Outdoor/Fine Tune/Sodium Lamp/Locked/Auto2 |  |  |  |  |  |  |
| Digital noise reduction |  | 2D/3D DNR |  |  |  |  |  |  |
| Smart IR |  | Support |  |  |  |  |  |  |
| Flip |  | Normal/Vertical/Horizontal/180°/90°Clockwise/90°Anti-clockwise |  |  |  |  |  |  |
| Dewarping |  | Support |  |  |  |  |  |  |
| HLC |  | Support |  |  |  |  |  |  |
| BLC |  | Support |  |  |  |  |  |  |
| Audio |  |  |  |  |  |  |  |  |
| Audio Compression | G.711 |  |  |  |  |  |  |  |
| Suppression | Supported |  |  |  |  |  |  |  |
| Sampling Rate | 8KHZ |  |  |  |  |  |  |  |
| Storage |  |  |  |  |  |  |  |  |
| Edge Storage | Micro SD, up to 256GB |  |  |  |  |  |  |  |
| Network Storage | ANR, NAS(NFS) |  |  |  |  |  |  |  |
| Network |  |  |  |  |  |  |  |  |
| Protocols | IPv4, IGMP, ICMP, ARP, TCP, UDP, DHCP, PPPoE, RTP, RTSP, RTCP, DNS, DDNS, NTP, FTP, UPnP, HTTP, HTTPS, SMTP, 802.1x, SNMP, QoS |  |  |  |  |  |  |  |
| Compatible Integration | ONVIF (Profile S, Profile G, Profile T), API |  |  |  |  |  |  |  |
| Client | EZStation EZView EZLive |  |  |  |  |  |  |  |
| Web Browser | Plug-in required live view: IE9+, Chrome 41 and below, Firefox 52 and below |  |  |  |  |  |  |  |
|  | Plug-in free live view: Chorme 57.0+, Firefox 58.0+, Edge 16+, Safari 11+ |  |  |  |  |  |  |  |
| Interface |  |  |  |  |  |  |  |  |
| Built-in Mic | Supported |  |  |  |  |  |  |  |
| Network | 1 RJ45 10M/100M Base-TX Ethernet |  |  |  |  |  |  |  |
| Certifications |  |  |  |  |  |  |  |  |
| Certifications |  | CE: EN 60950-1 UL: UL60950-1 FCC: FCC Part 15 |  |  |  |  |  |  |
| General |  |  |  |  |  |  |  |  |
| Power | DC 12V±25%, PoE (IEEE 802.3af) |  |  |  |  |  |  |  |
|  | Power consumption: Max 8.16W |  |  |  |  |  |  |  |
| Dimensions (Ø x H) | Φ129 x 110mm (Ø5.08” x 4.3”) |  |  |  |  |  |  |  |
| Net Weight | 0.82kg (1.8lb) |  |  |  |  |  |  |  |
| Material | Metal |  |  |  |  |  |  |  |
| Working Environment | -40°C ~ 60°C (-40°F ~ 140°F), Humidity: ≤95% RH (non-condensing) |  |  |  |  |  |  |  |
| Strorage Environment | -40°C ~ 60°C (-40°F ~ 140°F), Humidity: ≤95% RH (non-condensing) |  |  |  |  |  |  |  |
| surge protection | 6KV |  |  |  |  |  |  |  |
| Reset Button | Supported |  |  |  |  |  |  |  |
| Ingress Protection | IP67 |  |  |  |  |  |  |  |

Dimensions

Accessories

| TR-JB03-H-IN | TR-JB07/WM03-F-IN | TR-WM03-B-IN | TR-A01-IN |
| --- | --- | --- | --- |
| TR-UP06-IN | TR-SE24-IN | TR-SE24-A-IN | TR-CM24-IN |

| Junction box | Pole mount |
| --- | --- |
| TR-JB03-H-IN+TR-A01-IN | TR-JB07/WM03-F-IN (TR-WM03-B-IN)+ TR-UP06-IN |
| Indoor pendent mount |  |
| TR-JB04-D-IN+TR-A01-IN+ TR-SE24-IN(TR-SE24-A-IN)+ TR-CM24-IN |  |

Zhejiang Uniview Technologies Co., Ltd.

Building No.10, Wanlun Science Park, Jiangling Road 88, Binjiang District, Hangzhou, Zhejiang, China (310051)

Email: overseasbusiness@uniview.com; globalsupport@uniview.com

http://www.uniview.com

©2019 Zhejiang Uniview Technologies Co., Ltd. All rights reserved.

*Product specifications and availability are subject to change without notice.